I	PURPOSE

The purpose of this procedure is to provide for a system and instructions, and to assign responsibilities for performing the final inspection.

II	APPLICATION

This procedure applies to all finished supplies and services issued by ManTech. This procedure concerns Production and Quality Assurance.

III	PROCEDURE

1.	General

1.1	All finished products are subjected to final inspection before they are shipped to customers.  Quality Assurance is responsible for final inspections and for the release of product for packaging and shipping.  The QA inspectors carry out the inspections.

1.2	It is the policy of ManTech to concentrate resources and attention on defect prevention rather than detection.  The verification effort is focused on control of processes and in-process inspections. By the time products are completed all their components, parts and subassemblies have normally been already verified through the program of receiving and in-process inspections.  The purpose of final inspection is to review the records of all preceding inspections and tests, and to complete the evidence of conformance by verifying those product characteristics that have not been yet inspected.

2.	Scope

2.1	The scope of final inspection comprises:

- Review of the work order to ascertain that all specified operations, processes, and in-process inspections are recorded in the test and inspection plan

- Review of material certificates, inspection reports, control charts, and other quality records established prior to or during production

- Visual inspection of product to ascertain that all specified operations are completed and to detect any visible quality problems

- Taking measurements and testing to complete the evidence of product conformance

- Recording the actual measurements and test results and results recorded

3.	Carrying Out the Inspection

3.1	For complex products, and in particular when functional testing is involved, inspectors are provided with checklists, inspection procedures, inspection report forms, and other appropriate instructions.

3.2	Products that pass all reviews, inspections, and testing are identified by sticker, tag marking.  Passed products are then moved to the area where finished products are staged for packaging and/or shipping or installation.

4.	Nonconforming Product

4.1	When a nonconforming product is identified or quality documents are incomplete, the QA inspector identifies the product with a sticker, tag or marking, prepares a nonconformance report, marks the nonconformance report number on the sticker, and moves the product to a designated area.  The nonconformance report is further processed per Standard Operating Procedure SOP-P-05, Control of Nonconforming Product.

5.	Release of Product

5.1	QA inspection has the authority to release a product with Customer Representative concurrence. Only products that are identified may be installed, packaged and/or shipped.

6.	Inspection Record

6.1	The inspector signs and dates on the line where the final inspection is called out to establish the final inspection record.  Any checklist or reports prepared during the inspection are also preserved.

IV	Associated Documents

SOP–Q-02, Quality System Documentation

SOP–Q-03, Quality Records

SOP–P-05, Control of Nonconforming Product